University of Minnesota and MN Space Grant Consortium

AEM 1905 Freshman Seminar: Fall 2010

Spaceflight with Ballooning

Team Project Documentation

Team Hindenburg

Written by:

Anthony Knutson

Alex Knutson-Smisek

Henry Kunerth

Chad Larkin

Eric Ly

12/07/2010

Revision C

Table of Contents

0.0	Team Assignments.......................................................................................................3

1.0	Introduction..................................................................................................................5

2.0	Mission Overview........................................................................................................5

3.0	Payload Design............................................................................................................7

4.0	Project Management....................................................................................................10

5.0	Project Budgets............................................................................................................15

6.0	Payload Photographs....................................................................................................16

7.0	Test Plan and Results...................................................................................................19

8.0	Expected Science Results.............................................................................................23

9.0	Launch and Recovery...................................................................................................26

10.0	Results and Analysis....................................................................................................29

  - Conclusions and Lessons Learned...............................................................................48

12.0	Citations and Acknowledgements……………………………………………………..50

13.0	Appendix: Program Listings........................................................................................51

# 0.0 Team Assignments

# Writing Assignments

# Team Hindenburg

Introduction						Henry Kunerth

Mission Overview					Henry Kunerth

Payload Design					Anthony Knutson

Project Management					Eric Ly

Project Budgets					Chad Larkin

Payload Photographs					Henry Kunerth

Test Plan and Results					Alex Knutson-Smisek

Expected Science Results				Alex Knutson-Smisek

Launch and Recovery					Eric Ly

Results and Analysis					Anthony Knutson

Conclusions and Lessons Learned   		   	Chad Larkin

# Oral Presentation Assignments

Conceptual Design Review (CDR)		Henry Kunerth	Alex Knutson-Smisek

Flight Readiness Review (FFR)		Chad Larkin		Eric Ly

# Payload Build Assignments

Overall team lead and ground-testing lead		Alex Knutson-Smisek

Flight computer (BalloonSat Easy) build		Anthony Knutson

Weather station build					Chad Larkin

Payload box build					Anthony Knutson

Photographer						Henry Kunerth

Programmer (of flight computer(s))			Anthony Knutson

Camera and Camera Experiment			Henry Kunerth

HOBO (payload “health” (internal temp)		Eric Ly

Bacteria Experiment					Alex Knutson-Smisek

# Launch day Assignments

Photographer						Henry Kunerth

Prediction/tracking assistant				Alex Knutson-Smisek

Balloon filling and release assistant			Eric Ly

Payload/stack handling specialist			Anthony Knutson

Recovery specialist					Chad Larkin

1.0 Introduction

Reaching the final frontier – space – has been a part of the American ethos since the space race. However, it has until recently been limited in participation to those involved in high budget, high tech governmental programs. In the last two decades this has changed. Now, thanks to creativity and innovation from both professionals and amateur enthusiasts, the ability to conduct a scientific mission that reaches near-space has been made possible to everyone.

First, let’s take a look at what near-space means. As the name would imply, near-space is a high altitude region (approximately 70,000 to 350,000 feet above sea level) which lies within the earth’s atmosphere, but still short of true “outer-space.” It is well below the realm of satellites, but high enough to mimic space-like conditions and even see the curvature of the earth. For these reasons, reaching near-space has in recent years become a popular scientific endeavor. Near-spaceflight missions allow amateurs and experts alike to experience a field that was once monopolized by highly funded agencies.

But how does one reach near-space? The simplest solution is through the use of high altitude weather balloons. A typical near-space mission will involve building one or more light payloads with various experimental instruments, a heater, and a tracking system on board and then tethering them to a weather balloon. The balloon will then ascend to heights of around 100,000 feet before the balloon expands to its maximum volume and bursts. At this point, the apparatus will be caught by its parachute and, after a turbulent fall, will descend back to earth to be recovered. With luck, the instruments will be intact and the collected data will be ready to be analyzed.

Today, near-space is a territory opened up to everyone. Anyone with an interest and ambition can, through a bit of work and creativity, gain access to the realm once controlled only by high-budget research programs. And on top of that, there is plenty of near-space left for the professionals too.

2.0 Mission Overview

The Team Hindenburg near-space mission will attempt to launch a payload with various instruments and experiments through the troposphere and into the thermosphere by use of weather balloon generated lift.

The payload will consist of a sealed Styrofoam box fitted to a 6x6x6 inch interior cavity with holes cut for the camera lens, and the power switches. Weight allowing, the box will be coated in an insulating material that will also attempt to shield the interior from cosmic radiation. Four lines will secure to payload to the balloon and the other payloads.

The primary experiment deals with the bacteria Serratia marcescens. This type of bacteria is sensitive to ultraviolet radiation as well as cosmic radiation, so by observing how it reacts in comparison to a control sample in a lab, as well as a control sample exposed to radiation in a lab, we will make inferences as to the radiation levels in near-space. To further this experiment, we will test two cultures of the bacteria, one contained within the box and the other tethered to the outside. This will serve to test how well the bacteria were able to survive the extremes of near space including an even larger amount of radiation.

Additionally, we will be including a Geiger counter to measure radiation levels, a weather station device which will collect data on the temperature, relative humidity, and pressure, and a GPS device for tracking which will be located elsewhere on the stack. The collected data will be processed and stored using a HOBO inflight computer and also sent to the ground via a Zigbee radio. Battery packs will supply the power to these devices. To prevent damage or malfunction caused by temperature drops, a resistive heater will warm the interior of the payload.

Finally, we will be positioning a Flip Video camera facing upward to monitor changes in balloon size as pressure decreases, the eventual balloon burst before the descent stage, and the performance of the parachute.

The goals of the project are to successfully launch and land the payload with the data systems intact and ready for analysis. Challenges faced are assuring that the payload stays tethered to the balloon through any winds or turbulence faced, making sure all systems function when exposed to low temperatures and pressures, as well as possible high cosmic radiation levels, and successfully tracking the payload so that it can be recovered. The payload and instruments have been designed to withstand turbulence, low temperatures and pressures, and radiation, and will undergo preflight testing to confirm their efficacy in such situations. Recovery will depend on proper use of the GPS in navigation. It will be aided by a siren attached to the balloon system which will sound during the entire flight.

We expect to confirm previous measurements of temperatures, pressures, humidity, and cosmic radiation levels at various levels of the atmosphere, document the effects of cosmic and UV radiation on bacteria and how it can be shielded, and explore how a weather balloon expands as air pressure lowers.

3.0 Payload Design

For our payload, we have several specific design requirements unique to our team. First is that we need to carry several agar plates to hold the bacteria we are flying. We are planning on flying at least two containers on the inside, one shielded from radiation with aluminum foil or similar shielding, and one left only to be protected by our box. Also we plan to have at least one plate on the outside that will be exposed to the cold and radiation. Because of the extra weight of the agar plates, out team is considering not flying a camera. Since our experiment involves how bacteria are affected by cosmic radiation in near space, we will need to fly a Geiger counter to measure levels of radiation.

Payload Layout

Figure 3.1:

This is how we expect the inside of our payload box to look.

Cover of Box

Equipment List

1"-thick Pink Styrofoam (enough for 6" x 6" x 6" box)

Tubing, rigging strings, key rings, zip ties, glue, tape

Heater circuit (battery pack listed separately)

3-pack 9-volt battery for heater

Weather station (will attach to outside of box)

BalloonSat Easy flight computer

9-volt battery for flight computer

Three Agar plates for organisms

Flip Ultra video camera

HOBO data logger for interior "health" (has temperature and relative humidity built in)

HOBO temperature probe for outside temperature

Geiger counter to measure cosmic radiation

Serratia marcescens (Bacteria)

Zigbee radio to send Geiger Counter data to the ground during flight

Aluminum foil (UV radiation shielding for some of the bacteria)

BalloonSat Easy Flight Computer

9 Volt battery pack

Functional Block Diagram

9 Volt battery pack for heater

Figure 3.2:

This diagram shows how all of the components in our box will be connected.

Key:

Orange arrows indicate flow of electricity.

Green arrows indicate flow of data.

Flip Video Camera

Petri Dishes (Bacteria)

Zigbee Radio

Geiger Counter

Heater

Hobo Data Logger (Internal Temp)

External Measurement Sensors

Weather Station

Module Build & Documentation Duties

Henry

Henry

Eric

Alex

Anthony

Anthony

Anthony

Chad

HOBO

Camera Experiment

Photographer

Flight Computer

Programmer

Bacteria Experiment

Box Construction

Weather Station

Operational Module

4.0 Project Management

Team Lead

Alex

Figure 4.1:

This details the major roles that every member of the team is responsible for. Each member must make sure their section is progressing as necessary.

Launch Day Duties

Figure 4.2:

This shows everyone’s responsibilities on launch day.  This is important to make sure all necessary parts are ready for launch.

October 2010

| # SUNDAY | # MONDAY | TUESDAY | WEDNESDAY | THURSDAY | FRIDAY | SATURDAY |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1 | 2 |
| 3 | 4 | 5 Conceptual Design Review Due | 6 | 7 Finished Weather Station | 8 | 9 |
| 10 | 11 Finished Flight Computer | 12Box module is constructed Flight computer and weather station is tested-pass | 13 | 14 Payload box is strength tested- pass | 15 Team Project Documentation Rev A Due | 16 |
| 17 | 18 | 19 Thermal Testing is conducted, Flight computer was not operational. Retest required | 20 | 21 Payload Passes Thermal Testing | 22 | 23 |
| 24 31 | 25 | 26 Flight Readiness Review Due | 27 Components are zipped tied to the box. | 28 Payload Weight in/turn in Deadline Yank Test Passed Box is complete Box is complete | 29 Weather was brisk, mid 40s, low to slight winds | 30Launch Day Launch was successful, was unable to reach recovery site due to time |

November 2010

| # SUNDAY | # MONDAY | TUESDAY | WEDNESDAY | THURSDAY | FRIDAY | SATURDAY |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 Received information about our box. Camera and Geiger information was not good | 3 Working on data from flight | 4 Redesign box for additional flight. Another camera is added and the bacteria is placed on top. | 5 Team Project Documentation Rev B Due | 6 Re-launch of our box with slight modifications |
| 7 | 8 | 9 | 10 | 11 Team meeting about data analysis | 12 | 13 |
| 14 | 15 | 16 Preliminary Data Analysis Due | 17 | 18 Team meeting to continue data analysis and ready presentation | 19 | 20 |
| 21 | 22 | 23 Final Team Presentation | 24 | 25 Team meeting to discuss roles for Rev C | 26 | 27 |
| 28 | 29 | 30 Team further discussed roles for Rev C |  |  |  |  |

December 2010

| # SUNDAY | # MONDAY | TUESDAY | WEDNESDAY | THURSDAY | FRIDAY | SATURDAY |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 | 3 | 4 Optional viewing of Hubble |
| 5 | 6 | 7 Team Project Documentation Rev C Due Work on Exhibit | 8 | 9 Possible team meeting to prepare for exhibit | 10 | 11 |
| 12 | 13 | 14 Public Exhibit Course Wrap up | 15 | 16 | 17 | 18 |
| 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| 26 | 27 | 28 | 29 | 30 | 31 |  |

5.0 Project Budgets

A summary of the weight and cost of every item that Team Hindenburg used on our box:

| Part Number | Name | Estimated Unit Mass (kg) | Actual Unit Mass (kg) | Unit Cost ($) |
| --- | --- | --- | --- | --- |
| 1 | Styrofoam Shell | 0.18 | 0.162 | 8.00 |
| 2 | Rigging (Tubes, glue etc…) | 0.05 | 0.060 | 5.00 |
| 3 | Heater Circuit (battery pack not included) | 0.27 | 0.028 | 5.00 |
| 4 | 3-pack 9-volt Heater Battery Pack | 0.15 | 0.150 | 6.00 |
| 5 | Weather Station | 0.015 | 0.015 | 40.00 |
| 6 | BalloonSatEasy Flight Computer | 0.033 | 0.033 | 30.00 |
| 7 | 9-volt Battery for Flight Computer | 0.046 | 0.046 | 2.00 |
| 8 | Holding Containers for Bacteria | 0.5 | 0.030 | 9.00 |
| 9 | Flip Ultra Video Camera | 0.175 | 0.175 | 170.00 |
| 10 | HOBO Data Logger and Temperature Probe | 0.048 | 0.049 | 159.00 |
| 11 | Bacteria with Growth Medium | 0.01 | 0.020 | N/A |
| 12 | Zigbee Radio | 0.126 | 0.126 | 300.00 |
| 13 | Geiger Counter (RM-60) | N/A | 0.185 | 150.00 |
| Total |  | 1.603 kg | 1.0795 kg | $884 |

Figure 5.1:

The total mass of our payload ended up being significantly less than what we were expecting because the bacteria containers did not weigh nearly as much as we first put into our budget. We purposely added this number that was higher than what we expected so that we would be more conscious of the total weight of our payload until we obtained all of the final components.

6.0 Payload Photographs

Completed weather station

Resistive Heater Circuit (no batteries attached).

Geiger Counter

Flip Video Camera

Completed Flight computer (wires going out of picture go to weather station, batteries and switch).

Team Hindenburg members, Eric, Chad and Alex constructing the flight computer and weather station.

This picture shows how the Serratia look after about five days of growth. This is how we expect our control bacterial plates to look when we are comparing them with our flight plates after the flight. Shows many colonies, all of which have red color.

Agar plate (BHI) inoculated with Serratia Marcescens. The bacteria were recently inoculated so they have not yet developed their red color.

Team Hindenburg’s completed box, nearly ready for first flight.

Freshman seminar stack at launch. Team Hindenburg’s payload is the only one still be held by those on the ground.

Completed payload (closed).

7.0 Test Plan and Results

Our team will test the structural integrity of our payload box while it is empty of all components but filled with sand so that we know that the box will be able to withstand the stresses of flight and landing. We will drop the box from the top of a staircase and film the result. We will also examine the box afterwards to confirm that it is is still fully intact and to determine if some of the parts of the box are too weak to withstand flight.

We will check the data from the HOBO to make sure that it is reporting temperatures that are comparable to the actual temperature of the environment. To further test the HOBO we will place the box in a cold environment. We will use the data from the HOBO to see if the box stayed warm enough to keep components operational with the heat provided by the heater. This alternate environment will serve to double check that the HOBO is functioning correctly.

Our team will check the circuits of the flight computer to make sure that all components are placed correctly, and that will not be any electrical shorts. We will further test the flight computer by interfacing it with the corresponding computer program, and run the test programs to test all of the circuits on the computer. Finally we will load the flight program onto the computer and make sure that it is running correctly by watching the change in LED patterns corresponding to the different directives ordered by the program.

We will test our Zigbee Radio by making sure that it is able to accurately transmit the voltages of our different instruments to the receiving radio.

The weather station will first be calibrated so that we know what measure of voltage output corresponds to actual temperature, pressure, and relative humidity. After this calibration phase we will test the weather station on a different occasion to be sure that it is still transmitting accurate voltages.

In deciding between a video and a still camera to bring on our flight, our team did tests to ensure that the image quality of the video camera would be good enough and to see if the still camera program would be to much of a hassle. These tests included filming from different distances to ensure the clarity of the image over distance and filming in varying light conditions to prepare for dimming as we rise. We have decided to use a video camera in flight and we will test to make sure it is able to take video while pointing through the hole in our box. We will also test the method for turning the camera on pre-flight.

Our auxiliary experiment involves testing to see how cosmic radiation will affect bacteria, and possibly cause mutations. Before the flight we plan on testing both the viability of the bacterial culture as well as the effects of UV radiation exposure in a lab. To test the viability we will grow the bacteria in the laboratory to make sure they form a healthy sample. After this we will expose the samples to UV radiation similar to the radiation dosage the cultures will receive in flight, but instead of the sun, we will be using a machine called an Ultraviolet Crosslinker. With this instrument we will be able to vary the amount of UV radiation exposure to form a hypothesis as to what will happen to the bacteria.

The final component for us to test is our box's heater. We will test the heater by turning it on and then exposing the box to a temperature that is approximately what it will experience on the flight. We will use data from the HOBO to ensure that the inside of the box stays warm enough to prevent the batteries from dying.

Finally, we will test all of the components together in the complete box. The test will simulate a flight so we can make sure that all components are working together before the actual flight. This will also serve as an opportunity for us to practice all of the necessary preflight procedures before the launch our box. The test must include all of the steps taken during a launch and during a flight, and if any problems arise, we will have a chance to fix them before flight.

Checklist for Component Testing:

- Strength of payload box- Following the structural integrity test, our box had no visible damage to either the Styrofoam shell or the tape-secured joints.

- HOBO (to measure interior and exterior temperatures)- Our HOBO performed as expected: (The decrease in outside temperature at 17:18 corresponds with the payload going into the cold soak chamber. The increase in temperature at 17:32 corresponds with it coming out of the cold soak.)

Figure 7.1: HOBO Testing Graph. This shows us that our HOBO took accurate data during testing

Figure 7.2:  This data from our flight computer shows that all sensors were functioning properly during testing.

Flight computer (records weather station data)/ Weather Station- Our flight computer and weather station also preformed as expected: In the below graph we can see that the payload was in the cold soak from time 10 to approximately 74 because of a slight decrease in temperature. At the end of the graph the raw measurement jumps to 256 because this is the default value the computer records, meaning this is when the computer was shut off.

- Zigbee radio (to transmit Geiger Counter data to the ground)- We did not do any preflight testing of the Zigbee as intended. We did not do so because the reliable track record of the radio made us trust its performance at altitude.

- Test bacterial viability and effects of UV radiation in lab- Preflight testing was not preformed due to time constraints. However, our control culture will be tested in the manner described above after flight operations end.

- Flip video camera- The video camera exhibited excellent quality when tested as described above. It also continued to record throughout the “cold soak” experiment.

- Heater- The heater maintained a comfortable operating temperature inside our payload throughout the “cold soak.” (See HOBO data above.)

- Complete payload testing- Complete payload testing went according to plan. All of our components worked together well.

Figure 7.4:

This is a frame of a video taken of our payload’s yank test. In this test we are making sure that the rigging is going to be strong enough to keep the payload attached even during the violent descent. Our payload passed this test as well.

Figure 7.3:

This is a picture from our payload drop test. The payload was dropped down a flight of stairs while filled with sand to test the structural integrity. The box passed this test.

8.0 Expected Science Results

Temperature

Our balloon will be rising through two layers of the atmosphere, the Troposphere, and then the Stratosphere. Because of the different characteristics of these layers we expect to see changes in our temperature data.

The Troposphere goes from ground level to about 45,000 ft. In this layer temperature decreases with altitude at the rate of approximately -3.6°F/1,000 ft. So as our balloon rises through this lower part of the atmosphere we expect to see a steadily decreasing temperature.

Next our box will enter the layer between the troposphere and the Stratosphere called the Tropopause. In this layer the temperature stays fairly constant at a low temperature, so we expect to see our temperature data level off for a short period of time.

Finally our bow will enter the Stratosphere which goes from about 45,000 to 160,000 ft. In this layer the temperature surprisingly increases with altitude at the rate of 1.6°F/1,000 ft. So while the balloon completes its climb in this layer we expect to see a steady increase in temperature, but it will not go above what the temperature was on the ground.

Figures 8.1-8.2: (Graphs from Paul Verhage and Lyndon State College: Atmospheric Sciences Department)

Pressure

Figure 8.3: (Graph from Lyndon State College: Atmospheric Sciences Department)

Pressure decreases exponentially with altitude. This is because the majority of the air molecules in the atmosphere are pulled to their lowest energy position, which is as close to the Earth's surface as possible. This means that near the surface the number of air molecules is high, and as you go farther from the surface the number of molecules decreases exponentially. So our graph of pressure will show an approximately exponential decrease in pressure as altitude increases.

Relative Humidity

Relative humidity is a calculation of the amount of water vapor in the air, it is found by taking the partial pressure of the water pressure over what the saturated water pressure would be. Relative humidity is affected by both temperature and pressure. An overall decrease in temperature and pressure as altitude increases causes the relative humidity to decrease with altitude. We expect our data will show an overall decrease in relative humidity, and that it will also tell us whether we passed through any cloud banks because these will show up as large spikes in relative humidity.

Figure 8.4: (From University of Wyoming Sounder Station in Chanhassen, MN)

It should be noted that this graph is plotted with altitude on the horizontal axis unlike our other graphs.

Cosmic Radiation

The upper atmosphere is more affected by solar radiation because there is less atmosphere to block the cosmic radiation. The components of solar radiation that we are interested in are ionizing radiation, which is composed of mainly gamma rays and beta particles. Our box will be using a Geiger counter to detect these two kinds of radiation. The atmosphere blocks or deflects quite a bit of this radiation from reaching the Earth's surface, so we expect that the amount of radiation will increase as our altitude increases because there will be less atmosphere to block it. It is possible that our bacteria may also be affected by Ultraviolet radiation, which also increases with altitude.

Figure 8.5: (Cosmic Radiation graph from Paul Verhage)

Bacterial Experiment

Our box has three petri dishes with the bacteria Serratia Marcescens inoculated on BHI (Brain-Heart Infusion). One dish is mounted on the outside of the box, the other two are on the inside with one covered in tin foil. We expect that some of these bacteria's DNA will be mutated by the radiation that they are receiving. The plates on the inside of the payload box will be exposed to cosmic radiation because the ionized radiation particles have so much energy that they are easily able to pass through material such as Styrofoam. The plate that is exposed on the outside of our box will receive this cosmic radiation as well, but it will also be exposed to Ultraviolet radiation. Ultraviolet radiation is just electromagnetic waves that have a slightly shorter wavelength than visible light. The ozone layer of our atmosphere normally shields us from the majority of this radiation, which originates at the sun. But as we climb higher in altitude the amount of UV radiation will increase, and this could possibly cause mutations in the outer bacteria as well as the cosmic radiation. So we expect more bacteria will be mutated in the container on the outside of the payload. This mutation is likely to occur on a part of the bacterial DNA that controls the production of the pigment Prodigiosin. When this part of the DNA is mutated the bacteria will grow white instead of with the characteristic red color. From the number of bacterial colonies that grow white we will be able to compare the amount of radiation that they were exposed to. We expect to see that the bacteria on the outside of the box will have the most mutations, the ones on the inside uncovered will have some mutations, and those covered in tin foil will most likely have few mutations. We hypothesize theses results because we expect the box and the tin foil to both provide some protection against radiation by being a physical barrier. The bacteria on the outside of our box will also be exposed to ultraviolet radiation, which may contribute to the number of mutations.

Figure 8.6:

This diagram shows the results that we expect to see from our bacteria experiment. The colonies that receive the mutation will develop white streaks that will grow out due to the mutation being passed on to the offspring of the original bacteria that have mutated. From this we hope to be able to take quantitative results based on the number of mutations per colony. Or the number of colonies that have mutated compared to those that didn’t.

9.0  Launch and Recovery

Team Hindenburg had their first flight on the 30th of October 2010. The launch location was Pierz, Minnesota, about 30 miles north of St. Cloud. More specifically, the launch was performed in the practice fields of the local high school. The weather that day was brisk at around 40 degrees. Fortunate, the sky was clear and the wind speed was low. The AEM 1905: Spaceflight with Ballooning class left the University of Minnesota campus at approximately 6:30 and arrive at the site around9:00. Once everyone disembarked, each team examined their box and made final preparations for launch. Team Hindenburg inspected if all the zip-ties were secured and if not, they were tighten. In addition, the heater was briefly tested to ensure it would operate. A final preparation Team Hindenburg made was attaching the bacteria to the box. Not too long after examining the box, Team Hindenburg’s zigbee was turned on. Soon each team had determined their box ready to fly and the stack was slowly put together. This was the time period that most team had some extra time to take pictures and document this event. As the stack was being completed, the balloon was filled and each team turned on the devices within each box. It took over ten minutes for the helium to create enough lift to carry the stack. Toward the end of filling the balloon, a hand warmer was placed in the box to provide additional heat and the box was sealed with shipping tape. The siren was then turned on and the stack was released in a hand over hand method. That meaning the balloon lifted one module at a time until everything was in the air. At the 10:00 the stack was released and it floated westward.

The recovery of this flight was somewhat of a disappointment because the class was unable to reach the recovery site. The flight had flown 40 mile past the target landing site of Barron, Wisconsin. Although, there was incredible luck with the final landing position because the general area was filled with lakes and other water features. Unfortunately, an observation of the video provided by the recovery team showed it could of have landed in a better location. The stack had landed in a tree at the edge of the woods bordering a farm. Luckily, the string from the balloon was close enough to the ground for someone to grab. The stack was tugged on repeatedly until it fell from the tree. An interesting note is that the siren failed at some point in the flight leading to some difficulty in finding the stack. The recovery team had to focus entirely on sight and GPS information. That brings up another point. The tracking was rather spotty for this flight and communication with the APRS was lost a few times. That granted it was extremely fortunate for us to get all the components back.

Due to some failures in equipment, Team Hindenburg was allowed to fly the preceding week on the 6th of November. The launch location this time was in a public park of the small town of Danbury, Minnesota. Danbury is south of Duluth by 50 miles. This flight experienced a rougher launch than the previous one. It was partly cloudy and was rather windy with temperatures roughly the same as first flight. The launch was also tougher due to the limited amount of people. Last time there was people to spare; however, this time everyone had multiple duties. Also complicating matters further was the fact there was two balloons at the same site. It was planned to have four balloons launched at the same time across the state and then have them communicate with each other. Only the first goal was accomplished. Another problem was the parachute being tangled up, that required additional resources to fix it. All of these problems lead to a rushed launch that took off after 8:00.

The recovery was also more interesting compared to the first flight. The stack landed in between the two tiny towns of Prairie Farm and Ridgeland in Wisconsin. The location is south of the target landing site of Barron, WI. As with last time, the stack landed in a tree. However this time it was much higher up and in some thick woods. It took over three hours to take down the entire stack down. The recovery team had to use a huge slingshot to shoot ropes over the lines and hope it would connect with the stack. The ropes being used were weighted down on one end with balls so it would fall back down. Once the rope finally caught on the stack, the tugging began. After a little tugging the line snapped leaving a portion of the stack still in the tree. Dr. Flaten had to stay behind to retrieve the remaining parts. It should also be noted that the siren was going on the whole time compared to last time with the failure.

10.0  Results and Analysis

Pressure

Figure 10.1

The analysis of the data obtained from the flight reflects the expected results of Team Hindenburg. We had predicted that the movement of pressure will follow scientific knowledge in that pressure will decrease as altitude increases. The inverse is also true. By observing the graph, it can be noted that the curves of the line are very similar to the graph in the expected results section. However, the curves of the ascent and descent are slightly different. While both curves follow the same shape, the descent curve is slightly lower. The pressure was lower on the descent journey, even if it was at the same altitude as the ascent journey. It is believed that the difference occurred from the fast descent, the sensor simply could not keep up with the changing variables.

Relative Humidity

The results indicate that as altitude increases, the relative humidity will decrease. Relative humidity is highest near the ground and the lowest at the highest altitude. These findings reflected the hypothesis of Team Hindenburg. Though research, it was also determined that relative humidity will spike when entering a cloud layer. The data somewhat displays a spike in the percentage of relative humidity. On the descent, relative humidity was at a higher percentage than at the same altitude on the ascent. From that information, it can be assumed that we entered into some clouds. Lastly, it should be noted that a significant amount of data points were dropped. Those dropped data points would explain the vertical line on the left side of the graph.

Figure 10.4

Figure 10.3

Figure 10.2

Temperature

Figure 10.5

This graph plots temperature in Fahrenheit relative to time in minutes. The data begins at balloon release. As one can easily see, the temperature initially plummets. This is caused by the tendency for temperature to decrease as one ascends through the troposphere. Next, we see a rise in temperature. This marks the balloon entering and ascending through the stratosphere. After the balloon reaches its maximum height, it bursts and falls back through the stratosphere, dropping in temperature again. This time the temperature plummets even lower, which could be explained by either a lag in box temperature relative to the external temperature (meaning that the box began much colder on the descent, so it was cooled even further) or the fact that additional wind chill could have caused further cooling. The temperature ascends again as the box falls through the troposphere, finally leveling out when it hits ground. As can be seen, the box experienced a rather severe range of temperatures, from around 40 degrees at the ground to close to -60 in the low stratosphere/high troposphere. The data was collected using the BalloonSatEasy (BSE) program and the onboard weather station.

Altitude vs Time

Figure 10.6

The graph above plots temperature in Fahrenheit in relationship to altitude. The graph clearly demonstrates two separate boomerang shaped curves: one indicating ascent temperature and the other descent temperature. It may be easier to analyze them if they are broken into separate ascent and descent graphs.

Altitude vs Time (Ascent)

Figure 10.7

The above graph shows the temperature relative to altitude during the ascent phase. The temperature was recorded by the onboard weather station and stored by the BSE program. It is important to note that an anomaly was discovered in the data from approximately the first 9000 ft, in which the temperature appeared to rise and then fall, so data from this section was included. The data for the rest of the ascent phase, however, matches our predictions accurately. We can see the temperature fall steadily as the balloon transitions through the troposphere, and then rise as it moves through the stratosphere. The highest point on the graph occurs just prior to burst.

Altitude vs Time (Descent)

Figure 10.8

After burst, the temperature falls dramatically. This indicates the balloon’s path through the stratosphere. The temperature climbs as it falls through the stratosphere, before leveling off near the ground.

HOBO Temperature Data

Figure 10.9

The graph above plots data collected from the HOBO temperature probes and transmitted through the Zigbee radio. One temperature probe (blue) was positioned outside of the box and the other probe placed inside. We can clearly see that as the temperature falls in the same manner as demonstrated in the BSE graphs above, the internal temperature remains slightly higher, at least until both temperatures level off after landing. The troubling aspect, however, is exactly how cold the interior of the box became: over 40 degrees below zero. While we would expect some cooling, the resistive heater should not have allowed for this low of a drop in temperature. This may indicate a failure by the heater or a failure to turn on the heater before launch. The second theory (failure to turn on the heater) seems more likely, as we would expect to see a sudden rise in interior temperature shortly after the heater was turned on. As we can see, this is not observed on the graph, so it is likely the heater was never turned on.

Balloon Size

In order to calculate the diameter of the balloon relative to its original diameter, we had to account for the fact that as the balloon increased in size so did its distance from the camera. This phenomenon made the balloon appear to be smaller than it actually. We used paper with even spacing to measure the balloons apparent diameter every 5 minutes, starting when the balloon launched.  Then we divided the apparent number of spacing but the original number to obtain an apparent relative size. Below is a diagram to help explain our geometric correction.

Figure 10.10

The blue line represents the plane in which the original radius lies and all subsequent balloon radii appear to lie. Using similar triangles we come up with the following equality:

Figure 10.11

We can solve this for R actual in terms of R apparent (Length and R original are know) to obtain a corrected relative radius. This larger the balloon got the more important this correction became.  The following graph shows that as time increases (and therefore balloon radius increases) so does the difference between R apparent and R actual.

R Actual

R Apparent

The red dots are R actual and the blue dots are R apparent. The units on the x-axis are minutes after launch and the units on the y-axis are actual radius divided by original radius.

Since we had the corrected radius we plotted relative radius vs. altitude and relative volume vs. altitude.

Figure 10.12

Radius

Figure 10.13

Radius

In both graphs the units of the x-axis are feet above sea level. In the graph of radius vs. altitude the units on the y-axis are radius divided by original radius. In the volume vs. altitude graph the units on the y-axis are (radius/original radius)^3 or volume divided by original volume.

Perhaps our most interesting analysis dealing with balloon size was to confirm the ideal gas law, PV=nRT. This relationship implies that volume is proportional to temperature and inversely proportional to pressure. This means that a graph of volume vs. temperature/pressure should produce a linear fit. The graph below shows that the relationships implied by the ideal gas law are valid.

Figure 10.14

The units on the x-axis are temperature (in Kelvin) divided by pressure (in mb). The units on the y-axis are volume divided by original volume.

Note: For our data we measured the temperature and pressure outside the balloon, not inside. However, the air inside the balloon is heated or cooled to match the outside temperature and as we predicted and team Galactic Gopher showed, pressure outside and inside the balloon are very close.

Also, Dr. James Flaten deserves credit for helping us understand the geometric corrections for balloon radius.

Balloon Pictures

These pictures are taken from our up facing Flip video camera. From them we examined the size of the balloon.

Figure 10.15

Radiation

The above graph shows the change in radiation as you go up in altitude. Our hypothesis going into the experiment was: “Radiation increases with an increase in altitude.” This hypothesis seemed to make sense because as you float higher up in the atmosphere, and the density decreases, so should the radiation shielding. The Geiger counts follow this trend up to about 70000 feet, at which point the radiation counts begin to decrease again. This may be because at altitudes of 60000-70000 feet, the high concentration of ozone scatters the radiation, increasing exposure from all directions. While above 70000 feet the ozone concentration is lower, so the overall exposure is less.

Bacterial Results

Bacteria colonies 1 day after inoculation

The bacteria Serratia marcescens were exposed to the radiation in near space on our payload as it rose through the stratosphere. Three petri dishes containing these bacteria were on our payload box, one on the outside, and two on the inside, one of which was covered in approximately four layers of tin foil. There were also three control plates of bacteria that were kept at room temperature and stored in the lab.

The bacteria were examined about 48 hours after the flight. The petri dish that was on the outside of the payload did not provide us with results because the growth agar on the shriveled due to the vibrations and landing.  Both of the petri dishes that were on the inside of the payload survived, and showed growth of small white colonies. The plates were allowed to grow for another 48 hours so that these white colonies could grow and be examined more easily.

This is the way the bacterial colonies looked on the morning of the flight

The white colonies did not look as we expected them to be with white streaks extending from the in a line in the colony. Instead the colonies were slightly smaller, and were completely white. This result brought about some questions about whether these colonies were actually mutated Serratia, of if they were some kind of contamination of other bacteria, or possibly some other unexpected result.  Whether the white colonies were an anomolous result or not is not completely clear, but we do have some evidence to show that the colonies were actually Serratia, and were possibly mutated:

White Colonies (possibly of Serratia)

A possible reason for the colonies being completely white instead of streaked is that the bacteria in a colony are constantly dividing.  Each single bacterium does not have a very long lifespan, from a few hours to a few days. If bacteria in the white colonies had been mutated then it is possible that the red colonies that hadn’t been mutated died out and the colonies that had mutated white passed on their white color to their offspring.

White colonies on streak plate

All three of our control plates did not have any white colonies form. This is an indication that at least something different must have occurred on the plates that went on the flight.

Control Plate (no white colonies)

Samples of the white colonies were taken and streaked onto clean agar plates. On these new plates the bacteria are allowed to grow as a test to see if they will have the white color indicating the mutation. These white colony samples showed many more white colonies forming on the streaked plate, but red colonies also grew. The expected results were that all colonies on the streaked plates would have the white mutant color. We reason that red colonies grew on these new plates because of a mechanism in bacteria called the “light repair mechanism.” Extended exposure to UV light, such as just sitting out in the sun, activates an enzyme in the bacteria that does a “spell check” of the bacteria’s DNA. This enzyme activates a repair mechanism which will correct the mutation in the Serratia’s DNA. We suspect that some of our mutant Serratia had repairs in its DNA, and that is why not all of the colonies on the streak plates were white. But because of this change back to the characteristic red color of Serratia we were able to know that the colonies that were white on the flight plates were actually colonies of Serratia. The streak plate bacteria also had the characteristic smell that growing Serratia produces further proof that the white colonies were Serratia.

- There was also a fairly high quantity of white colonies that formed on these streaked plates. The plates that we used as control did not have any white colonies form, showing that spontaneous mutations in Serratia (ones that might occur just to the randomness of the bacteria replication, and not from radiation exposure) are a very rare occurrence. Because of this comparison we can see that the streak plate Serratia are not growing normally, and the reason for this abnormality is likely due to the radiation exposure.

Red colonies on streak plate

We encountered an unexpected result when we also made a streak plate of red colonies from the flight plates that also contained the white mutant colonies. We expected this would show that the red colonies did not contain any mutations, and would just continue growing red on these plates. But the red colonies that were restreaked actually formed some white colonies as well. This result is surprising because it shows that even though the colonies only appeared red on the flight plates, mutations had probably occurred in them as w ell causing some of these mutated bacteria to form white colonies.

11.0  Conclusions and Lessons Learned

On our payload’s two trips to near space we had a few things go wrong and many things go right. Despite some minor technical setbacks, we managed to recover some cool and, most importantly, usable data.

Flight 1:

On this flight we had on board the standard pressure, relative humidity and internal/external temperature sensors, a “looking up” camera to record the expansion of Galactic Gophers balloon, three Petri dishes of the bacteria Serratia marcescens, one on the outside of the payload and two on the inside, and a Geiger counter. As well as our flight computer and weather station, HOBO, heater and Zigbee radio. All of our equipment stayed functional throughout the flight however, due to a mix-up in the Zigbee identification numbers, the readings from our Geiger counter were not transmitted to the ground. Also, because our payload was not far enough away from that of Galactic Gophers, we were not able to observe the expansion of their small balloon. Our bacterial experiment worked fairly well, however at some point during the flight or landing the bacterial culture on the outside of our payload collapsed into an unusable mess. Fortunately the bacteria on the inside of the payload were still usable. We also did get usable data from both the weather station and the HOBO.

Flight 2:

On this flight we had the same instruments as on the first flight with the addition of a “looking down” camera to view the ground as the payload rose. Because we were assured a low spot on the stack this time, we decided to take measurements of the expansion of the main balloon with our other camera, in addition to observing Galactic Gopher’s balloon. This time our Geiger data was successfully transmitted to the ground and, again, our HOBO and weather station data was fairly complete and usable. Also, we were able to see both Galactic Gopher’s balloon and the main balloon to monitor the expansion rate. On the down side, our heater failed shortly into the flight and the internal temperature of our payload bottomed out at a little under    -40 degrees Fahrenheit. Luckily all of our equipment survived the extremely cold temperatures. Our bacteria however did not. The culture attached to the outside of the payload was completely dried up and the cultures on the inside were barely usable. We think that this was caused by a lower relative humidity on this flight, in conjunction with the heater failure. We also discovered after recovering the payload that our “looking down” camera had not recorded any video throughout the entire flight. We think that the camera may not have been turned on in the preflight setup. Because there were fewer people to help prepare the stack, Alex, the only member from our team to go along on the second flight, was not the one to get our payload ready for flight. We think that in the confusion, whoever prepared our box failed to press the power button.

With data from both flights we have confirmed the hypotheses that we went into the flight with. Namely that temperature and relative humidity vary with altitude, pressure decreases and balloon size increases with altitude, radiation increases with altitude, and that bacteria are affected by exposure to radiation.

From all of the experiences we had with the successes and failures of the equipment and experiments on these flights, we have learned to check equipment before flight and to take charge of preflight preparation of the payload to try and prevent simple malfunctions that may “balloon” into larger problems mid flight. Our words of wisdom to those hoping to partake in a near-space mission are:

- Communication is key to ensuring effective and efficient teamwork

- Don't be afraid to think outside the box and pursue areas you are interested in, don't just do what everyone else is doing

- Just because a piece of equipment has worked well in the past, always double check to make sure it is in working condition before flight

12.0 Citations and Acknowledgements

Internet Sources:

- Temperature vs. Altitude graphs from Paul Verhage and Lyndon State College: Atmospheric Sciences Department (http://apollo.lsc.vsc.edu)

- Pressure Graph from Lyndon State College: Atmospheric Sciences Department (http://apollo.lsc.vsc.edu)

- Relative Humidity Graph was created from data posted by the University of Wyoming Sounder Station in Chanhassen, MN (http://www.weather.uwyo.edu/upperair/sounding.html) (click on MPX on map)

- Radiation Graph taken from Paul Verhage

Personal Sources:

- Dr. James Flaten: For organizing the ballooning program, as well as providing help and advice throughout the process.

- Cait Mantych: For supporting the ballooning program as well as providing transportation on flight day.

- Seth Frick: For holding regular office hours each week that our team used extensively, as well as always providing knowledgeable advice on any of our questions. Seth was also an instrumental part of our second flight.

- Kyle Marek-Spartz: For also providing regular office hours, and for his help in preparing and launching our payload on the second flight.

- Joey Senkyr: For providing transportation on the first flight, and his brilliant orange pants.

- Rebecca Merica, Kathy Fryxell, Marty Wolf and Pat Goodman: These four Microbiology Lab Professors provided us with support, materials, and lab equipment that was absolutely essential to performing our bacteria experiment. They also offered their knowledge and opinions during the interpretation of our experiment’s results.

13.0  Appendix: Program Listings

Flight Computer

' File: FreshmanSeminarFlightCodever3 (used Fall 2010)

symbol record=w0  'This is the section where the variables are declared

symbol index=w1

symbol value=b4

BalloonSat:

symbol Max_ADC = 2                     ' maximum adc channel usedstarting with 0

symbol Mission_Delay = 15000           ' length of pause in mission loop 15 seconds

Mission_Prep:

i2cslave %10100000,i2cfast,i2cword     ' set memory speed to 400 kHz

if pin7 = 1 then Download_Data 	     'and one word records

flashed:					'this section is the section that waits

high  3					'for commit pin to be pulled

pause 1000					'the flahser is also in this section

low 3						' it flashes at a specific rate

pause 1000

if pin7=0 then flashed

Mission:			' will change pattern of flashing when data is being taken

gosub Analog                              ' collect analog voltages

write 0,record                            ' store the number of records collected

gosub On_Flash                      ' pause.....

goto Mission                              ' ....before starting all over

Analog:

for index = 0 to Max_ADC             ' loop for number of analog voltages to record

readadc index,value                ' get next adc value

gosub Record_Data                  ' go store the value

next                                 ' until last voltage is recorded

return                               ' return to main mission loop

Record_Data:

if record = 3000 then End_Mission  ' check that aren't writing too many records to memory

record = record + 1                 ' increment record number

low 0                               ' unwrite protect memory

writei2c record,(value)             ' write the next record to memory

pause 10                            ' wait 10 ms for write

high 0                             ' write protect memory

return                             ' return to the calling calling subroutine

On_Flash:

high 3

pause 1000				'flash twice than a long pause

low 3

pause 500

high 3

pause 1000

low 3

pause 12500

return

Download_Data:

sertxd ( cr,lf)

sertxd (cr,lf)

sertxd ("Welcome to Balloonsat Easy Data Download Routine ",cr,lf)

sertxd ("Data Download will be in 3 seperate interface sections",Cr,lf)

sertxd ("After the data section is completed copy input buffer to a text file",Cr,lf)

sertxd ("Then clear input buffer and replace the commit pin when read back resumes remove commit pin ",Cr,lf)

Sertxd (" Clear the input buffer and replace the commit pin")

gosub flasher

sertxd ("Data section 1 of 3",Cr,lf)

for record = 1 to 1000

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record + 1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record +1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,Cr,lf)                         ' serial out the data record

next

sertxd ("Data section 1 of 3 Complete",Cr,lf)

gosub flasher

sertxd (Cr,lf)

sertxd ("Data section 2 of 3 ",Cr,lf)

for record = 1000 to 2000

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record + 1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record +1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,Cr,lf)                         ' serial out the data record

next

sertxd ("Data section 2 of 3 Complete",Cr,lf)

gosub flasher							' waits to replace the commit pin

' than remove commit pin

sertxd (Cr,lf)

sertxd ("Data section 3 of 3 ",Cr,lf)

for record = 1999 to 3000

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record + 1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record +1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,Cr,lf)                         ' serial out the data record

next

sertxd ("Data section 3 of 3 Complete",Cr,lf)

gosub flasher

sertxd (Cr,lf)

sertxd ("Data Download Complete",Cr,lf)                  ' until last data record read out

gosub LT_down

LT_down:

high 3					'flash 3 times than pause

Pause 1000					'signifies completed

low 3 					' download data

pause 500

high 3

pause 1000

low 3

pause 500

high 3

pause 1000

low 3

pause 10000

Goto LT_down

flasher:

pause 1000

if pin7=1 then flasher			'waits the for commit pin

return

End_Mission:	'this is if data was recorded during the whole flight

low 3 		' this shows that the memory is full

pause 10000		' and that the flight computer functioned properly for the flight

high 3

pause 1000

goto End_Mission

'this program has a problem

' it writes the record location to internal memory not to the

' 16 bit 1 word memory chip on the balloonsat easy 2.0 flight computer board

'there forethe data should exist for any

end                                           ' end of mission

Our payload included a Paul Verhage designed flight computer. This computer was programmed to take voltage data from our weather station which was measuring temperature, pressure and relative humidity. The computer’s program told it to record the measurements from each of these instruments every 15 seconds. This data is stored into the computer’s memory so that it can be retrieved after the flight. The program is set up so that it does not start recording data until the computer is turned on and the commit pin is pulled. This ensures that the computers are much more difficult to start accidentally, so they will only start recording just prior to a flight. One thing to note about this program is that the when the commit pin is reinserted into the flight computer it is possible for the computer to overwrite the data that has been previously recorded; the computer also will start overwriting the oldest data points if it is left recording longer than its memory can hold.