'Code previously called "TeacherSeminarFlightCodeLive.bas"

'also used for AEM 1905 as "BSE_WeatherFlightCodeLive.bas"

'"live" means data is written to window as well as being saved, for testing

symbol record=w0  'This is the section where the variables are declared

symbol index=w1

symbol value=b4

BalloonSat:

symbol Max_ADC = 2                     ' maximum adc channel usedstarting with 0

symbol Mission_Delay = 15000           ' length of pause in mission loop 15 seconds

Mission_Prep:

i2cslave %10100000,i2cfast,i2cword     ' set memory speed to 400 kHz

if pin7 = 1 then Download_Data 	     'and one word records

sertxd ("Pull pin to take data.",Cr,lf)                         ' serial out the data record

flashed:					'this section is the section that waits

high  3					'for commit pin to be pulled

pause 1000					'the flahser is also in this section

low 3						' it flashes at a specific rate

pause 1000

if pin7=0 then flashed

Mission:			' will change pattern of flashing when data is being taken

gosub Analog                              ' collect analog voltages

write 0,record                            ' store the number of records collected

gosub On_Flash                      ' pause.....

goto Mission                              ' ....before starting all over

Analog:

for index = 0 to Max_ADC             ' loop for number of analog voltages to record

readadc index,value                ' get next adc value

gosub Record_Data                  ' go store the value

next                                 ' until last voltage is recorded

sertxd (Cr,lf)                         ' serial out the data record

return                               ' return to main mission loop

Record_Data:

if record = 3000 then End_Mission  ' check that aren't writing too many records to memory

record = record + 1                 ' increment record number

low 0                               ' unwrite protect memory

writei2c record,(value)             ' write the next record to memory

sertxd (#value,",")                         ' serial out the data record

pause 10                            ' wait 10 ms for write

high 0                             ' write protect memory

return                             ' return to the calling calling subroutine

On_Flash:

high 3

pause 1000				'flash twice than a long pause

low 3

pause 500

high 3

pause 1000

low 3

pause 12500

return

Download_Data:

sertxd (Cr,lf)

sertxd ("Download Data",Cr,lf)

for record = 1 to 1000

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record + 1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record +1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,Cr,lf)                         ' serial out the data record

next

gosub flasher

sertxd (Cr,lf)

sertxd ("Download Data",Cr,lf)

for record = 1000 to 2000

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record + 1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record +1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,Cr,lf)                         ' serial out the data record

next

gosub flasher							' waits to replace the commit pin

' than remove commit pin

sertxd (Cr,lf)

sertxd ("Download Data",Cr,lf)

for record = 1999 to 3000

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record + 1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,",")                         ' serial out the data record

record = record +1

readi2c record,(value)                       ' read the recorded record

sertxd (#value,Cr,lf)                         ' serial out the data record

next

gosub flasher

sertxd (Cr,lf)

sertxd ("Download Data",Cr,lf)                  ' until last data record read out

gosub LT_down

LT_down:

high 3					'flash 3 times than pause

Pause 1000					'signifies completed

low 3 					' download data

pause 500

high 3

pause 1000

low 3

pause 500

high 3

pause 1000

low 3

pause 10000

Goto LT_down

flasher:

pause 1000

if pin7=1 then flasher			'waits the for commit pin

return

End_Mission:	'this is if data was recorded during the whole flight

low 3 		' this shows that the memory is full

pause 10000		' and that the flight computer functioned properly for the flight

high 3

pause 1000

goto End_Mission

'this program has a problem

' it writes the record location to internal memory not to the

' 16 bit 1 word memory chip on the balloonsat easy 2.0 flight computer board

'there forethe data should exist for any

end                                           ' end of mission